# Complete CCNA Study Guide with Lab Tasks

## Table of Contents

- Phase 1: The Absolute Basics

- Phase 2: Local Area Networks (LANs)

- Phase 3: Routing & WANs

- Phase 4: Network Services & Security

- Phase 5: Advanced & Specialized Topics

# Phase 1: The Absolute Basics

## 1.1 Networking Fundamentals

### Key Concepts

Network Devices:

- Hub: Layer 1 device, creates single collision domain, broadcasts to all ports

- Switch: Layer 2 device, learns MAC addresses, creates separate collision domains per port

- Router: Layer 3 device, routes between networks using IP addresses, separates broadcast domains

Collision Domain vs Broadcast Domain:

- Collision Domain: Area where data collisions can occur (hubs create one large collision domain)

- Broadcast Domain: Area where broadcast frames are propagated (routers separate broadcast domains)

### Lab Task 1.1: Basic Network Connectivity

Topology:

PC1 ---- Switch ---- PC2

Configuration Steps:

Physical Connections:

Connect PC1 to Switch Fa0/1

Connect PC2 to Switch Fa0/2

Use copper straight-through cables

PC Configuration:

PC1:

IP Address: 192.168.1.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

PC2:

IP Address: 192.168.1.20

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

Testing:

PC1> ping 192.168.1.20

PC1> arp -a

Expected Results:

- Ping should be successful

- ARP table should show PC2's MAC address

## 1.2 OSI & TCP/IP Models

### OSI Model (7 Layers)

| Layer | Name | Function | Devices/Protocols |
| --- | --- | --- | --- |
| 7 | Application | User interface | HTTP, FTP, SMTP |
| 6 | Presentation | Data formatting, encryption | SSL, JPEG, ASCII |
| 5 | Session | Session management | NetBIOS, SQL |
| 4 | Transport | End-to-end delivery | TCP, UDP |
| 3 | Network | Logical addressing, routing | IP, ICMP, Routers |
| 2 | Data Link | Physical addressing, switching | Ethernet, Switches |
| 1 | Physical | Transmission medium | Cables, Hubs |

### TCP/IP Model (4 Layers)

| Layer | OSI Equivalent | Function |
| --- | --- | --- |
| Application | 5-7 | Application services |
| Transport | 4 | End-to-end communication |
| Internet | 3 | Routing |
| Network Access | 1-2 | Physical network access |

### Lab Task 1.2: Protocol Analysis

Objective: Understand how different protocols work at different layers

Steps:

- Set up the same topology as Task 1.1

- Use Packet Tracer simulation mode

- Send a ping from PC1 to PC2

- Analyze the packet at each layer:

  - Layer 1: Electrical signals

  - Layer 2: Ethernet frame (MAC addresses)

  - Layer 3: IP packet (IP addresses)

  - Layer 4: ICMP message

Commands for Analysis:

PC1> ping 192.168.1.20

PC1> tracert 192.168.1.20

## 1.3 IP Addressing (IPv4/IPv6)

### IPv4 Addressing

Address Classes:

- Class A: 1.0.0.0 to 126.0.0.0 (/8)

- Class B: 128.0.0.0 to 191.255.0.0 (/16)

- Class C: 192.0.0.0 to 223.255.255.0 (/24)

Private Address Ranges:

- Class A: 10.0.0.0/8

- Class B: 172.16.0.0/12

- Class C: 192.168.0.0/16

Special Addresses:

- Loopback: 127.0.0.0/8

- APIPA: 169.254.0.0/16

- Multicast: 224.0.0.0/4

### Subnetting

CIDR Notation:

- /24 = 255.255.255.0

- /25 = 255.255.255.128

- /26 = 255.255.255.192

- /27 = 255.255.255.224

Subnetting Formula:

- Number of subnets = 2^n (where n = borrowed bits)

- Hosts per subnet = 2^h - 2 (where h = host bits)

### Lab Task 1.3: Subnetting Practice

Scenario: Subnet 192.168.1.0/24 into 4 subnets

Solution:

- Borrowed bits: 2 (2^2 = 4 subnets)

- New subnet mask: /26 (255.255.255.192)

- Subnets:

  - 192.168.1.0/26 (192.168.1.1-62)

  - 192.168.1.64/26 (192.168.1.65-126)

  - 192.168.1.128/26 (192.168.1.129-190)

  - 192.168.1.192/26 (192.168.1.193-254)

Topology:

PC1(192.168.1.10/26) ---- Switch ---- PC2(192.168.1.70/26)

Testing:

PC1> ping 192.168.1.70

This should fail because they're in different subnets without a router.

### IPv6 Addressing

Address Types:

- Unicast: One-to-one communication

- Multicast: One-to-many communication

- Anycast: One-to-nearest communication

Address Format:

- 128-bit address

- 8 groups of 4 hexadecimal digits

- Example: 2001:db8:85a3::8a2e:370:7334

Special IPv6 Addresses:

- Loopback: ::1/128

- Link-local: fe80::/10

- Unique local: fc00::/7

# Phase 2: Local Area Networks (LANs)

## 2.1 Ethernet & Switching

### Ethernet Standards

| Standard | Speed | Cable Type |
| --- | --- | --- |
| 10BASE-T | 10 Mbps | Cat 3 UTP |
| 100BASE-TX | 100 Mbps | Cat 5 UTP |
| 1000BASE-T | 1 Gbps | Cat 5e/6 UTP |
| 10GBASE-T | 10 Gbps | Cat 6a UTP |

### Switch Operation

MAC Address Table:

- Switches learn MAC addresses dynamically

- Default aging time: 300 seconds

- CAM table stores MAC-to-port mappings

Frame Processing:

- Learn: Source MAC address

- Forward: Based on destination MAC

- Flood: If destination unknown

- Filter: If source and destination on same port

### Lab Task 2.1: Switch MAC Address Learning

Topology:

PC1 ---- Fa0/1 [Switch] Fa0/2 ---- PC2

Fa0/3

|

PC3

Switch Configuration:

Switch> enable

Switch# show mac address-table

Switch# clear mac address-table dynamic

Testing Sequence:

- Clear MAC table

- Send ping from PC1 to PC2

- Check MAC table after each step

- Observe learning process

Expected Commands:

PC1> ping 192.168.1.20

Switch# show mac address-table

Switch# show mac address-table dynamic interface fa0/1

## 2.2 VLANs (Virtual LANs)

### VLAN Concepts

Benefits:

- Logical segmentation

- Improved security

- Broadcast domain control

- Simplified management

VLAN Types:

- Data VLAN: User traffic

- Voice VLAN: VoIP traffic

- Management VLAN: Switch management

- Native VLAN: Untagged traffic on trunk

### VLAN Configuration

Basic VLAN Commands:

Switch(config)# vlan 10

Switch(config-vlan)# name Sales

Switch(config)# vlan 20

Switch(config-vlan)# name Engineering

Switch(config)# interface fa0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Switch(config)# interface fa0/2

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 20

### Lab Task 2.2: VLAN Configuration

Topology:

PC1(VLAN 10) ---- Fa0/1 [Switch1] Fa0/24 ---- Fa0/24 [Switch2] Fa0/1 ---- PC3(VLAN 10)

PC2(VLAN 20) ---- Fa0/2                                        Fa0/2 ---- PC4(VLAN 20)

Switch1 Configuration:

Switch1(config)# vlan 10

Switch1(config-vlan)# name Sales

Switch1(config)# vlan 20

Switch1(config-vlan)# name Engineering

Switch1(config)# interface fa0/1

Switch1(config-if)# switchport mode access

Switch1(config-if)# switchport access vlan 10

Switch1(config)# interface fa0/2

Switch1(config-if)# switchport mode access

Switch1(config-if)# switchport access vlan 20

Switch1(config)# interface fa0/24

Switch1(config-if)# switchport mode trunk

Switch2 Configuration:

Switch2(config)# vlan 10

Switch2(config-vlan)# name Sales

Switch2(config)# vlan 20

Switch2(config-vlan)# name Engineering

Switch2(config)# interface fa0/1

Switch2(config-if)# switchport mode access

Switch2(config-if)# switchport access vlan 10

Switch2(config)# interface fa0/2

Switch2(config-if)# switchport mode access

Switch2(config-if)# switchport access vlan 20

Switch2(config)# interface fa0/24

Switch2(config-if)# switchport mode trunk

Verification Commands:

Switch# show vlan brief

Switch# show interfaces trunk

Switch# show interfaces fa0/1 switchport

## 2.3 Trunking (802.1Q)

### Trunking Concepts

802.1Q Frame Format:

- 4-byte VLAN tag inserted into Ethernet frame

- VLAN ID field: 12 bits (4094 possible VLANs)

- Native VLAN: Untagged traffic (default VLAN 1)

Trunk Negotiation Protocols:

- DTP (Dynamic Trunking Protocol): Cisco proprietary

- Manual configuration: Recommended for security

### Trunking Configuration

Trunk Port Configuration:

Switch(config)# interface fa0/24

Switch(config-if)# switchport mode trunk

Switch(config-if)# switchport trunk allowed vlan 10,20

Switch(config-if)# switchport trunk native vlan 99

DTP Modes:

- switchport mode trunk: Permanent trunk

- switchport mode access: Permanent access

- switchport mode dynamic auto: Trunk if neighbour desires

- switchport mode dynamic desirable: Actively negotiate trunk

### Lab Task 2.3: Advanced Trunking

Objective: Configure trunk with specific VLANs and native VLAN

Configuration:

Switch1(config)# interface fa0/24

Switch1(config-if)# switchport trunk encapsulation dot1q

Switch1(config-if)# switchport mode trunk

Switch1(config-if)# switchport trunk allowed vlan 10,20,30

Switch1(config-if)# switchport trunk native vlan 99

Switch1(config-if)# switchport nonegotiate

Verification:

Switch# show interfaces fa0/24 trunk

Switch# show interfaces fa0/24 switchport

Switch# show dtp interface fa0/24

## 2.4 Spanning Tree Protocol (STP)

### STP Concepts

Purpose:

- Prevent Layer 2 loops

- Provide redundancy

- Automatic failover

STP States:

- Blocking: Receives BPDUs, blocks data

- Listening: Processes BPDUs, no MAC learning

- Learning: Builds MAC table, no forwarding

- Forwarding: Normal operation

- Disabled: Administratively down

Port Roles:

- Root Port: Best path to root bridge

- Designated Port: Sends BPDUs on segment

- Blocked Port: Prevents loops

### STP Configuration

Root Bridge Configuration:

Switch(config)# spanning-tree vlan 1 priority 4096

Switch(config)# spanning-tree vlan 1 root primary

Port Priority Configuration:

Switch(config)# interface fa0/1

Switch(config-if)# spanning-tree port-priority 128

Switch(config-if)# spanning-tree cost 19

### Lab Task 2.4: STP Convergence

Topology:

Switch1 (Root)

/              \

Switch2  --------  Switch3

Configuration:

# Make Switch1 root bridge

Switch1(config)# spanning-tree vlan 1 priority 4096

# Verify STP topology

Switch# show spanning-tree

Switch# show spanning-tree summary

Switch# show spanning-tree interface fa0/1

Testing:

- Verify port states

- Disconnect primary link

- Observe reconvergence

- Check new port states

## 2.5 VTP (VLAN Trunking Protocol)

### VTP Concepts

VTP Modes:

- Server: Can create/modify/delete VLANs

- Client: Cannot modify VLAN database

- Transparent: Forwards VTP messages, independent database

VTP Requirements:

- Same VTP domain name

- Same VTP password (if configured)

- Trunk links between switches

### VTP Configuration

Server Configuration:

Switch(config)# vtp domain COMPANY

Switch(config)# vtp mode server

Switch(config)# vtp password cisco123

Switch(config)# vtp version 2

Client Configuration:

Switch(config)# vtp domain COMPANY

Switch(config)# vtp mode client

Switch(config)# vtp password cisco123

### Lab Task 2.5: VTP Domain Setup

Topology:

Switch1(Server) ---- Switch2(Client) ---- Switch3(Transparent)

Configuration Steps:

- Configure VTP domain on all switches

- Set modes appropriately

- Create VLANs on server

- Verify propagation to client

- Verify transparent mode behavior

Verification:

Switch# show vtp status

Switch# show vtp counters

Switch# show vlan brief

# Phase 3: Routing & WANs

## 3.1 Basic Router Configuration

### Router Boot Process

- POST (Power-On Self Test)

- Bootstrap program

- IOS loading

- Configuration loading

### Initial Router Configuration

Basic Setup:

Router> enable

Router# configure terminal

Router(config)# hostname R1

R1(config)# enable password cisco

R1(config)# enable secret class

R1(config)# line console 0

R1(config-line)# password console

R1(config-line)# login

R1(config-line)# exit

R1(config)# line vty 0 4

R1(config-line)# password telnet

R1(config-line)# login

R1(config-line)# exit

R1(config)# service password-encryption

R1(config)# banner motd #Unauthorized Access Prohibited#

Interface Configuration:

R1(config)# interface gigabitethernet 0/0

R1(config-if)# ip address 192.168.1.1 255.255.255.0

R1(config-if)# description LAN Interface

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface serial 0/0/0

R1(config-if)# ip address 10.1.1.1 255.255.255.252

R1(config-if)# description WAN Interface

R1(config-if)# clock rate 64000

R1(config-if)# no shutdown

### Lab Task 3.1: Router Initial Configuration

Topology:

PC1 ---- Gi0/0 [R1] S0/0/0 ---- S0/0/0 [R2] Gi0/0 ---- PC2

192.168.1.0/24    10.1.1.0/30      192.168.2.0/24

R1 Configuration:

R1(config)# interface gi0/0

R1(config-if)# ip address 192.168.1.1 255.255.255.0

R1(config-if)# no shutdown

R1(config)# interface s0/0/0

R1(config-if)# ip address 10.1.1.1 255.255.255.252

R1(config-if)# clock rate 64000

R1(config-if)# no shutdown

R2 Configuration:

R2(config)# interface gi0/0

R2(config-if)# ip address 192.168.2.1 255.255.255.0

R2(config-if)# no shutdown

R2(config)# interface s0/0/0

R2(config-if)# ip address 10.1.1.2 255.255.255.252

R2(config-if)# no shutdown

Verification:

R1# show ip interface brief

R1# show running-config

R1# ping 10.1.1.2

R1# show cdp neighbors

## 3.2 Static Routing

### Static Route Concepts

Route Types:

- Directly Connected: Interface networks

- Static: Manually configured

- Dynamic: Learned via routing protocols

Administrative Distance:

- Connected: 0

- Static: 1

- EIGRP: 90

- OSPF: 110

- RIP: 120

### Static Route Configuration

Standard Static Route:

Router(config)# ip route destination_network subnet_mask next_hop_ip

Router(config)# ip route 192.168.2.0 255.255.255.0 10.1.1.2

Default Route:

Router(config)# ip route 0.0.0.0 0.0.0.0 next_hop_ip

Summary Route:

Router(config)# ip route 172.16.0.0 255.255.0.0 10.1.1.2

### Lab Task 3.2: Static Routing Configuration

Topology:

PC1 ---- [R1] ---- [R2] ---- [R3] ---- PC2

192.168.1.0/24  |   10.1.1.0/30  |   192.168.3.0/24

|                |

192.168.2.0/24   10.1.1.4/30

R1 Static Routes:

R1(config)# ip route 192.168.2.0 255.255.255.0 10.1.1.2

R1(config)# ip route 192.168.3.0 255.255.255.0 10.1.1.2

R1(config)# ip route 10.1.1.4 255.255.255.252 10.1.1.2

R2 Static Routes:

R2(config)# ip route 192.168.1.0 255.255.255.0 10.1.1.1

R2(config)# ip route 192.168.3.0 255.255.255.0 10.1.1.6

R3 Static Routes:

R3(config)# ip route 192.168.1.0 255.255.255.0 10.1.1.5

R3(config)# ip route 192.168.2.0 255.255.255.0 10.1.1.5

R3(config)# ip route 10.1.1.0 255.255.255.252 10.1.1.5

Verification:

R1# show ip route

R1# show ip route static

PC1> tracert 192.168.3.10

## 3.3 Dynamic Routing (RIP, EIGRP, OSPF)

### RIP (Routing Information Protocol)

RIP Characteristics:

- Distance vector protocol

- Hop count metric (max 15)

- Updates every 30 seconds

- Uses broadcast/multicast

RIP Configuration:

Router(config)# router rip

Router(config-router)# version 2

Router(config-router)# network 192.168.1.0

Router(config-router)# network 10.0.0.0

Router(config-router)# no auto-summary

### EIGRP (Enhanced Interior Gateway Routing Protocol)

EIGRP Characteristics:

- Cisco proprietary (now open standard)

- Advanced distance vector

- Fast convergence

- Unequal cost load balancing

EIGRP Configuration:

Router(config)# router eigrp 100

Router(config-router)# network 192.168.1.0 0.0.0.255

Router(config-router)# network 10.1.1.0 0.0.0.3

Router(config-router)# no auto-summary

### OSPF (Open Shortest Path First)

OSPF Characteristics:

- Link state protocol

- Fast convergence

- Hierarchical design with areas

- Cost metric based on bandwidth

OSPF Configuration:

Router(config)# router ospf 1

Router(config-router)# network 192.168.1.0 0.0.0.255 area 0

Router(config-router)# network 10.1.1.0 0.0.0.3 area 0

### Lab Task 3.3: Dynamic Routing Protocols

Topology for RIP:

PC1 ---- [R1] ---- [R2] ---- [R3] ---- PC2

RIP Configuration:

# On all routers

Router(config)# router rip

Router(config-router)# version 2

Router(config-router)# network [directly_connected_networks]

Router(config-router)# no auto-summary

OSPF Configuration:

# On all routers

Router(config)# router ospf 1

Router(config-router)# network 0.0.0.0 255.255.255.255 area 0

Verification Commands:

Router# show ip route

Router# show ip protocols

Router# show ip rip database

Router# show ip ospf database

Router# show ip eigrp topology

## 3.4 WAN Technologies

### WAN Connection Types

Serial Connections:

- Point-to-point links

- Frame Relay

- ATM

- MPLS

PPP (Point-to-Point Protocol):

Router(config)# interface serial 0/0/0

Router(config-if)# encapsulation ppp

Router(config-if)# ppp authentication chap

Router(config-if)# username remote_router password shared_secret

Frame Relay:

Router(config)# interface serial 0/0/0

Router(config-if)# encapsulation frame-relay

Router(config-if)# frame-relay map ip 192.168.1.2 102 broadcast

### Lab Task 3.4: PPP Configuration

Topology:

R1 ---- PPP Link ---- R2

R1 Configuration:

R1(config)# username R2 password cisco123

R1(config)# interface s0/0/0

R1(config-if)# encapsulation ppp

R1(config-if)# ppp authentication chap

R1(config-if)# ip address 10.1.1.1 255.255.255.252

R1(config-if)# no shutdown

R2 Configuration:

R2(config)# username R1 password cisco123

R2(config)# interface s0/0/0

R2(config-if)# encapsulation ppp

R2(config-if)# ppp authentication chap

R2(config-if)# ip address 10.1.1.2 255.255.255.252

R2(config-if)# no shutdown

Verification:

R1# show ppp multilink

R1# debug ppp authentication

## 3.5 NAT (Network Address Translation)

### NAT Types

Static NAT:

- One-to-one mapping

- Permanent translation

Dynamic NAT:

- Pool of public addresses

- First-come, first-served

PAT (Port Address Translation):

- Many-to-one using port numbers

- Most common implementation

### NAT Configuration

Static NAT:

Router(config)# ip nat inside source static 192.168.1.10 203.0.113.10

Router(config)# interface gi0/0

Router(config-if)# ip nat inside

Router(config)# interface s0/0/0

Router(config-if)# ip nat outside

Dynamic NAT:

Router(config)# ip nat pool PUBLIC 203.0.113.1 203.0.113.10 netmask 255.255.255.0

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

Router(config)# ip nat inside source list 1 pool PUBLIC

PAT Configuration:

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

Router(config)# ip nat inside source list 1 interface s0/0/0 overload

Router(config)# interface gi0/0

Router(config-if)# ip nat inside

Router(config)# interface s0/0/0

Router(config-if)# ip nat outside

### Lab Task 3.5: NAT/PAT Configuration

Topology:

PC1 ---- [Router] ---- Internet

PC2 ----    NAT    ---- (ISP Router)

192.168.1.0/24  |  203.0.113.1/30

Router Configuration:

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

Router(config)# ip nat inside source list 1 interface s0/0/0 overload

Router(config)# interface gi0/0

Router(config-if)# ip nat inside

Router(config)# interface s0/0/0

Router(config-if)# ip nat outside

Verification:

Router# show ip nat translations

Router# show ip nat statistics

Router# clear ip nat translation *

# Phase 4: Network Services & Security

## 4.1 DHCP (Dynamic Host Configuration Protocol)

### DHCP Process (DORA)

- Discover: Client broadcasts DHCP discover

- Offer: Server responds with IP offer

- Request: Client requests specific IP

- Acknowledge: Server confirms assignment

### DHCP Configuration

Router as DHCP Server:

Router(config)# ip dhcp excluded-address 192.168.1.1 192.168.1.10

Router(config)# ip dhcp pool LAN_POOL

Router(dhcp-config)# network 192.168.1.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.1.1

Router(dhcp-config)# dns-server 8.8.8.8 8.8.4.4

Router(dhcp-config)# lease 7

DHCP Relay Agent:

Router(config)# interface gi0/0

Router(config-if)# ip helper-address 192.168.100.10

### Lab Task 4.1: DHCP Server Configuration

Topology:

PC1 ---- [Router/DHCP Server] ---- [ISP Router]

PC2 ----   192.168.1.1/24     ---- Internet

DHCP Configuration:

Router(config)# ip dhcp excluded-address 192.168.1.1 192.168.1.10

Router(config)# ip dhcp pool INTERNAL

Router(dhcp-config)# network 192.168.1.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.1.1

Router(dhcp-config)# dns-server 8.8.8.8

Router(dhcp-config)# domain-name company.com

Router(dhcp-config)# lease 0 12 0

Client Configuration:

- Set PCs to obtain IP automatically

- Release and renew IP addresses

Verification:

Router# show ip dhcp binding

Router# show ip dhcp pool

Router# show ip dhcp conflict

PC> ipconfig /all

PC> ipconfig /release

PC> ipconfig /renew

## 4.2 Telnet & SSH

### Telnet Configuration

Basic Telnet Setup:

Router(config)# line vty 0 4

Router(config-line)# password telnet123

Router(config-line)# login

Router(config-line)# transport input telnet

### SSH Configuration

SSH Setup:

Router(config)# hostname R1

R1(config)# ip domain-name company.com

R1(config)# crypto key generate rsa

R1(config)# ip ssh version 2

R1(config)# username admin privilege 15 secret admin123

R1(config)# line vty 0 4

R1(config-line)# transport input ssh

R1(config-line)# login local

### Lab Task 4.2: SSH Configuration

Objective: Configure secure remote access to router

Configuration:

Router(config)# hostname R1

R1(config)# ip domain-name lab.local

R1(config)# crypto key generate rsa general-keys modulus 1024

R1(config)# ip ssh time-out 120

R1(config)# ip ssh authentication-retries 3

R1(config)# username admin privilege 15 secret cisco123

R1(config)# line vty 0 4

R1(config-line)# transport input ssh

R1(config-line)# login local

R1(config-line)# exec-timeout 10 0

Testing SSH Access:

PC> ssh -l admin 192.168.1.1

Verification:

R1# show ssh

R1# show ip ssh

R1# show users

R1# show login

## 4.3 Access Control Lists (ACLs)

### ACL Types

Standard ACLs (1-99, 1300-1999):

- Filter based on source IP only

- Applied close to destination

Extended ACLs (100-199, 2000-2699):

- Filter based on source, destination, protocol, port

- Applied close to source

Named ACLs:

- More descriptive than numbered

- Allow editing of individual entries

### Standard ACL Configuration

Numbered Standard ACL:

Router(config)# access-list 10 permit 192.168.1.0 0.0.0.255

Router(config)# access-list 10 deny 192.168.2.10 0.0.0.0

Router(config)# access-list 10 permit any

Router(config)# interface gi0/1

Router(config-if)# ip access-group 10 out

Named Standard ACL:

Router(config)# ip access-list standard SALES_FILTER

Router(config-std-nacl)# permit 192.168.10.0 0.0.0.255

Router(config-std-nacl)# deny any

Router(config)# interface gi0/0

Router(config-if)# ip access-group SALES_FILTER in

### Extended ACL Configuration

Numbered Extended ACL:

Router(config)# access-list 101 deny tcp 192.168.1.0 0.0.0.255 192.168.2.0 0.0.0.255 eq 80

Router(config)# access-list 101 permit tcp any any eq 443

Router(config)# access-list 101 permit icmp any any

Router(config)# access-list 101 deny ip any any

Router(config)# interface gi0/0

Router(config-if)# ip access-group 101 in

Named Extended ACL:

Router(config)# ip access-list extended WEB_FILTER

Router(config-ext-nacl)# deny tcp 192.168.1.0 0.0.0.255 any eq 80

Router(config-ext-nacl)# permit tcp 192.168.1.0 0.0.0.255 any eq 443

Router(config-ext-nacl)# permit icmp any any echo-reply

Router(config-ext-nacl)# permit icmp any any unreachable

Router(config-ext-nacl)# deny ip any any log

Router(config)# interface gi0/0

Router(config-if)# ip access-group WEB_FILTER out

### Lab Task 4.3: Comprehensive ACL Implementation

Topology:

Sales_PC ---- [Router] ---- Web_Server

(192.168.10.10)  |      (192.168.20.10)

|

Engineering_PC ---

(192.168.10.20)

Requirements:

- Sales can access web server HTTP only

- Engineering can access all services

- Block all other traffic

- Allow ICMP for troubleshooting

ACL Configuration:

Router(config)# ip access-list extended DEPARTMENT_POLICY

Router(config-ext-nacl)# remark Allow Sales HTTP access

Router(config-ext-nacl)# permit tcp host 192.168.10.10 host 192.168.20.10 eq 80

Router(config-ext-nacl)# remark Allow Engineering full access

Router(config-ext-nacl)# permit ip host 192.168.10.20 192.168.20.0 0.0.0.255

Router(config-ext-nacl)# remark Allow ICMP for troubleshooting

Router(config-ext-nacl)# permit icmp 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255

Router(config-ext-nacl)# remark Deny all other traffic

Router(config-ext-nacl)# deny ip any any log

Router(config)# interface gi0/0

Router(config-if)# ip access-group DEPARTMENT_POLICY out

Verification:

Router# show access-lists

Router# show access-lists DEPARTMENT_POLICY

Router# show ip interface gi0/0

Testing:

Sales_PC> ping 192.168.20.10

Engineering_PC> telnet 192.168.20.10

## 4.4 Additional Network Services

### NTP (Network Time Protocol)

NTP Client Configuration:

Router(config)# clock timezone PST -8

Router(config)# clock summer-time PDT recurring

Router(config)# ntp server 129.6.15.28

Router(config)# ntp server 132.163.4.101

NTP Server Configuration:

Router(config)# ntp master 3

Router(config)# ntp source gi0/0

### SNMP (Simple Network Management Protocol)

SNMPv2c Configuration:

Router(config)# snmp-server community public ro

Router(config)# snmp-server community private rw

Router(config)# snmp-server location "Network Closet A"

Router(config)# snmp-server contact "admin@company.com"

SNMPv3 Configuration:

Router(config)# snmp-server group ADMINS v3 priv

Router(config)# snmp-server user admin ADMINS v3 auth sha cisco123 priv aes 128 cisco456

Router(config)# snmp-server host 192.168.1.100 version 3 priv admin

### Syslog Configuration

Syslog Setup:

Router(config)# logging 192.168.1.50

Router(config)# logging trap informational

Router(config)# logging source-interface gi0/0

Router(config)# service timestamps log datetime msec

Router(config)# logging buffered 4096 debugging

### Lab Task 4.4: Network Services Integration

Objective: Configure multiple network services for network management

NTP Configuration:

Router(config)# ntp server pool.ntp.org

Router(config)# clock timezone EST -5

Router(config)# service timestamps log datetime msec

Syslog Configuration:

Router(config)# logging host 192.168.1.100

Router(config)# logging trap notifications

Router(config)# logging source-interface gi0/0

SNMP Configuration:

Router(config)# snmp-server community monitor ro

Router(config)# snmp-server location "Main Office Router"

Router(config)# snmp-server contact "Network Admin"

Verification:

Router# show ntp status

Router# show clock

Router# show logging

Router# show snmp

# Phase 5: Advanced & Specialized Topics

## 5.1 Wireless LAN (Standards, Security)

### Wireless Standards

| Standard | Frequency | Max Speed | Range |
| --- | --- | --- | --- |
| 802.11a | 5 GHz | 54 Mbps | 35m |
| 802.11b | 2.4 GHz | 11 Mbps | 140m |
| 802.11g | 2.4 GHz | 54 Mbps | 140m |
| 802.11n | 2.4/5 GHz | 600 Mbps | 250m |
| 802.11ac | 5 GHz | 6.93 Gbps | 35m |
| 802.11ax | 2.4/5 GHz | 9.6 Gbps | 35m |

### Wireless Security

Security Protocols:

- WEP: Deprecated, easily cracked

- WPA: TKIP encryption, PSK or Enterprise

- WPA2: AES encryption, most common

- WPA3: Enhanced security, SAE authentication

### Wireless Controller Configuration

Basic Wireless Setup:

Controller(config)# wlan create 10 CORPORATE ssid CORP-WIFI

Controller(config-wlan)# security wpa psk set-key ascii 0 StrongPassword123

Controller(config-wlan)# security wpa akm psk

Controller(config-wlan)# no shutdown

Controller(config)# interface create VLAN10 10

Controller(config)# interface vlan 10 dhcp required

### Lab Task 5.1: Wireless Network Configuration

Topology:

Laptop ~~~ Access Point ---- Switch ---- Router

|                |           |

Controller      DHCP Server  Internet

Access Point Configuration:

AP(config)# interface dot11radio 0

AP(config-if)# ssid CORPORATE

AP(config-ssid)# authentication open

AP(config-ssid)# wpa-psk ascii 0 SecureWiFi2023

AP(config)# interface dot11radio 0

AP(config-if)# encryption mode ciphers aes-ccm

Verification:

AP# show dot11 associations

AP# show interface dot11radio 0

Controller# show wlan summary

## 5.2 EtherChannel (PAgP, LACP)

### EtherChannel Concepts

Benefits:

- Increased bandwidth

- Load balancing

- Redundancy

- No spanning tree blocking

Protocols:

- PAgP: Cisco proprietary

- LACP: IEEE 802.3ad standard

### EtherChannel Configuration

LACP Configuration:

Switch(config)# interface range fa0/1-2

Switch(config-if-range)# channel-group 1 mode active

Switch(config-if-range)# exit

Switch(config)# interface port-channel 1

Switch(config-if)# switchport mode trunk

PAgP Configuration:

Switch(config)# interface range fa0/3-4

Switch(config-if-range)# channel-group 2 mode desirable

Switch(config-if-range)# exit

Switch(config)# interface port-channel 2

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Manual EtherChannel:

Switch(config)# interface range gi0/1-2

Switch(config-if-range)# channel-group 3 mode on

Switch(config-if-range)# exit

Switch(config)# interface port-channel 3

Switch(config-if)# switchport mode trunk

Switch(config-if)# switchport trunk allowed vlan 10,20,30

### Lab Task 5.2: EtherChannel Implementation

Topology:

Switch1

/      \

Po1/      \Po1

/        \

Switch2 ---- Switch3

Po2

Switch1 Configuration:

Switch1(config)# interface range fa0/1-2

Switch1(config-if-range)# channel-group 1 mode active

Switch1(config-if-range)# exit

Switch1(config)# interface range fa0/3-4

Switch1(config-if-range)# channel-group 2 mode active

Switch1(config-if-range)# exit

Switch1(config)# interface port-channel 1

Switch1(config-if)# switchport mode trunk

Switch1(config)# interface port-channel 2

Switch1(config-if)# switchport mode trunk

Verification:

Switch# show etherchannel summary

Switch# show etherchannel port-channel

Switch# show interfaces port-channel 1

Switch# show pagp counters

Switch# show lacp counters

Load Balancing Configuration:

Switch(config)# port-channel load-balance src-dst-mac

## 5.3 First-Hop Redundancy Protocols (HSRP, VRRP, GLBP)

### HSRP (Hot Standby Router Protocol)

HSRP Concepts:

- Cisco proprietary

- Active/Standby model

- Virtual IP and MAC address

- Default priority: 100

HSRP Configuration:

Router1(config)# interface gi0/0

Router1(config-if)# ip address 192.168.1.2 255.255.255.0

Router1(config-if)# standby 1 ip 192.168.1.1

Router1(config-if)# standby 1 priority 110

Router1(config-if)# standby 1 preempt

Router1(config-if)# standby 1 track serial 0/0/0

Router2(config)# interface gi0/0

Router2(config-if)# ip address 192.168.1.3 255.255.255.0

Router2(config-if)# standby 1 ip 192.168.1.1

Router2(config-if)# standby 1 priority 90

### VRRP (Virtual Router Redundancy Protocol)

VRRP Configuration:

Router1(config)# interface gi0/0

Router1(config-if)# ip address 192.168.1.2 255.255.255.0

Router1(config-if)# vrrp 1 ip 192.168.1.1

Router1(config-if)# vrrp 1 priority 110

Router1(config-if)# vrrp 1 preempt

Router2(config)# interface gi0/0

Router2(config-if)# ip address 192.168.1.3 255.255.255.0

Router2(config-if)# vrrp 1 ip 192.168.1.1

Router2(config-if)# vrrp 1 priority 90

### Lab Task 5.3: HSRP Implementation

Topology:

PC1 ---- Switch ---- Router1 (Active)

PC2 ----    |   ---- Router2 (Standby)

Default Gateway: 192.168.1.1 (Virtual IP)

Router1 (Primary) Configuration:

R1(config)# interface gi0/0

R1(config-if)# ip address 192.168.1.10 255.255.255.0

R1(config-if)# standby version 2

R1(config-if)# standby 1 ip 192.168.1.1

R1(config-if)# standby 1 priority 110

R1(config-if)# standby 1 preempt delay minimum 30

R1(config-if)# standby 1 track gi0/1 60

R1(config-if)# standby 1 authentication md5 key-string hsrp123

Router2 (Secondary) Configuration:

R2(config)# interface gi0/0

R2(config-if)# ip address 192.168.1.20 255.255.255.0

R2(config-if)# standby version 2

R2(config-if)# standby 1 ip 192.168.1.1

R2(config-if)# standby 1 priority 90

R2(config-if)# standby 1 preempt delay minimum 30

R2(config-if)# standby 1 authentication md5 key-string hsrp123

Verification:

Router# show standby

Router# show standby brief

Router# debug standby events

Testing Failover:

- Shutdown active router's interface

- Verify standby takes over

- Check PC connectivity maintained

- Bring interface back up

- Verify preemption if configured

## 5.4 Advanced Switching Features

### Port Security

Port Security Configuration:

Switch(config)# interface fa0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport port-security

Switch(config-if)# switchport port-security maximum 2

Switch(config-if)# switchport port-security mac-address sticky

Switch(config-if)# switchport port-security violation restrict

Violation Actions:

- Protect: Drops packets, no notification

- Restrict: Drops packets, sends SNMP trap

- Shutdown: Disables port (default)

### DHCP Snooping

DHCP Snooping Configuration:

Switch(config)# ip dhcp snooping

Switch(config)# ip dhcp snooping vlan 10,20

Switch(config)# interface fa0/1

Switch(config-if)# ip dhcp snooping trust

Switch(config)# ip dhcp snooping information option

### Dynamic ARP Inspection (DAI)

DAI Configuration:

Switch(config)# ip arp inspection vlan 10

Switch(config)# interface fa0/1

Switch(config-if)# ip arp inspection trust

Switch(config)# ip arp inspection validate src-mac dst-mac ip

### Lab Task 5.4: Advanced Security Features

Topology:

DHCP_Server ---- Trusted_Port [Switch] Untrusted_Ports ---- PCs

|

Trusted_Port ---- Router

Security Configuration:

# Enable DHCP Snooping

Switch(config)# ip dhcp snooping

Switch(config)# ip dhcp snooping vlan 10

# Configure trusted ports

Switch(config)# interface fa0/24

Switch(config-if)# ip dhcp snooping trust

# Configure port security

Switch(config)# interface fa0/1

Switch(config-if)# switchport port-security

Switch(config-if)# switchport port-security maximum 1

Switch(config-if)# switchport port-security mac-address sticky

Switch(config-if)# switchport port-security violation shutdown

# Enable DAI

Switch(config)# ip arp inspection vlan 10

Switch(config)# interface fa0/24

Switch(config-if)# ip arp inspection trust

Verification:

Switch# show ip dhcp snooping binding

Switch# show port-security

Switch# show ip arp inspection

## 5.5 Troubleshooting

### Common Troubleshooting Commands

Layer 1 (Physical):

Router# show interfaces

Router# show controllers serial 0/0/0

Switch# show interfaces status

Switch# show power inline

Layer 2 (Data Link):

Switch# show mac address-table

Switch# show spanning-tree

Switch# show vlan brief

Switch# show interfaces trunk

Layer 3 (Network):

Router# show ip route

Router# show ip arp

Router# ping 192.168.1.1

Router# traceroute 192.168.1.1

Layer 4+ (Transport/Application):

Router# show ip sockets

Router# telnet 192.168.1.1 80

PC> netstat -an

PC> nslookup google.com

### Systematic Troubleshooting Approach

- Define the Problem

  - What is not working?

  - When did it stop working?

  - What changed recently?

- Gather Information

  - Network topology

  - Error symptoms

  - Affected devices/users

- Consider Possibilities

  - Layer-by-layer analysis

  - Common failure points

  - Recent changes

- Create Action Plan

  - Start with most likely cause

  - Least disruptive tests first

  - Document findings

- Implement and Test

  - Make one change at a time

  - Test after each change

  - Roll back if unsuccessful

- Monitor and Document

  - Verify solution works

  - Document resolution

  - Implement preventive measures

### Lab Task 5.5: Comprehensive Network Troubleshooting

Scenario: Users in VLAN 10 cannot access the internet, but VLAN 20 users can.

Troubleshooting Steps:

- Verify Physical Layer:

- Switch# show interfaces fa0/1

- Switch# show interfaces status

- Check Data Link Layer:

- Switch# show vlan brief

- Switch# show interfaces fa0/1 switchport

- Switch# show mac address-table vlan 10

- Verify Network Layer:

- Router# show ip route

- Router# show ip interface brief

- PC> ipconfig /all

- PC> ping 192.168.10.1

- Test Connectivity:

- PC> ping 8.8.8.8

- PC> nslookup google.com

- Router# show ip nat translations

- Check ACLs and Policies:

- Router# show access-lists

- Router# show ip interface gi0/0

Common Issues and Solutions:

| Issue | Symptoms | Solution |
| --- | --- | --- |
| Wrong VLAN assignment | Can't reach gateway | Verify VLAN configuration |
| Missing route | Can reach local, not remote | Add static/dynamic routes |
| ACL blocking | Selective connectivity issues | Review ACL rules |
| NAT problems | Can't reach internet | Check NAT configuration |
| DHCP issues | No IP address | Verify DHCP server/relay |

## Summary and Next Steps

### CCNA Exam Topics Coverage

This study guide covers all major CCNA exam topics:

✅ Network Fundamentals (20%)

- OSI and TCP/IP models

- Ethernet concepts

- IPv4/IPv6 addressing and subnetting

✅ Network Access (20%)

- VLANs and trunking

- EtherChannel

- Wireless concepts

✅ IP Connectivity (25%)

- Routing concepts

- Static and dynamic routing

- First-hop redundancy protocols

✅ IP Services (10%)

- NAT/PAT

- DHCP

- NTP, SNMP, Syslog

✅ Security Fundamentals (15%)

- ACLs

- Wireless security

- Port security

✅ Automation and Programmability (10%)

- Network management

- APIs and configuration management

### Recommended Lab Practice Schedule

Week 1-2: Foundation

- Complete all Phase 1 labs

- Focus on basic connectivity and addressing

Week 3-4: Switching

- Master VLAN and trunking concepts

- Practice STP scenarios

Week 5-6: Routing

- Static and dynamic routing protocols

- NAT/PAT configuration

Week 7-8: Services and Security

- DHCP, ACLs, and network services

- Wireless configuration

Week 9-10: Advanced Topics

- EtherChannel and HSRP

- Comprehensive troubleshooting

### Additional Resources

- Cisco Packet Tracer: Essential for lab practice

- Cisco Documentation: Latest configuration guides

- Practice Exams: Test knowledge retention

- Video Training: Supplement hands-on practice

### Final Exam Preparation Tips

- Hands-on Practice: Configuration is key

- Understand Concepts: Don't just memorize commands

- Troubleshooting Skills: Practice systematic approach

- Time Management: Know command shortcuts

- Documentation: Keep configuration templates